TASK-2

Arduino Uno R3

- CPU: 8-bit AVR, single core, 16 MHz

- RAM: 2 KB SRAM

- Power: 5 V over USB. Draws roughly 45–50 mA.

- Typical use case: reading sensors.

Raspberry Pi 4 Model B

- CPU: quad-core, 64-bit, 1.5 GHz.

- RAM: 2 GB / 4 GB / 8 GB.

- Power: 5 V

- Typical use case: vision and compute — running OpenCV on a camera feed.

Comparison

- Determinism vs. capability: the Uno executes one loop with nothing else competing for the CPU, so a pulse timed to the microsecond stays timed to the microsecond. The Pi's Linux scheduler can preempt your process at any moment, which makes it far more capable but unreliable for hard real-time control.

- Boot and shutdown: the Uno is running code within milliseconds of power-up and can be unplugged mid-operation without harm. The Pi takes ~30 seconds to boot and can corrupt its SD card if you cut power without shutting down.

- Memory scale changes what's possible: 2 KB of SRAM rules out storing an image, so the Uno can only handle scalar sensor readings. Gigabytes of RAM let the Pi hold video frames and neural network weights.

- Power budget: ~50 mA against ~1 A is a 20× gap, which decides battery life on a mobile robot. The Uno sleeps at microamps; the Pi has no comparable low-power state.

- Programming model: the Uno is flashed with a single C/C++ sketch over USB, with no OS and no multitasking. The Pi runs Python, C++, or anything else, several programs at once, with networking, drivers, and package managers available.

- Cost and expendability: roughly ₹500–900 for an Uno versus ₹4,000–8,000 for a Pi 4, so the Uno is what you put on the prototype you expect to short out.