Task 3

Model chosen: Decision tree

Approximate number of parameters: 50-100

Type of input: A short vector of numeric features.

Type of output: A single class label.

Why this model is suitable for edge devices:

- It fits in kilobytes. A few hundred bytes of thresholds sits comfortably in the SRAM of even a small microcontroller, where a neural network would not.

- Inference is only comparisons. No multiplication and no floating point is required, so an 8-bit microcontroller runs it natively without an accelerator.

- Only part of the model runs per prediction. A depth-5 tree visits 5 nodes rather than all 30, so cost scales with depth rather than total size — unlike a neural network, where every parameter is used in every forward pass.

- Latency is predictable and bounded, since the worst case is exactly the depth of the tree. This matters for real-time control loops, where variable delay is worse than a slightly longer fixed delay.

- Power draw is negligible because the work per inference is so small. On a battery-powered sensor node, transmitting the result costs far more energy than computing it.

- It is inspectable. The tree can be printed and read directly, which makes debugging on hardware much easier when there is no debugger available.